EXP NO: 1

Estimation of Characteristic Impedance (Zo), and Propagation Constant (γ) of a Transmission Line from Primary Constants.

Test Case 1:

Investigating the significance of Characteristic Impedance (Zo), and Propagation Constant

(γ) of a Transmission Line using the following parameters

Code:

clear; clc;

R=10;L=0.003;f=2*10^3;G=4*(10^-6);C=16*(10^-6);

w=2*%pi*f;

Z=R+(%i*w*L);

Y=G+(%i*w*C);

Zo=sqrt(Z/Y);

C=round(real(Zo));

D=round(imag(Zo));

printf('-Zo = %f + j(%f)ohms\n',C,D);

P=sqrt(Z*Y);

a=real(P);

a1=round(a*10000)/10000;

printf('-Attenuation constant a = %f neper/km\n',a1);

b=imag(P);

b1=round(b*10000)/10000;

printf('-Phase constant b = %f radians/km',b1);

OUTPUT:

| Simulation results |  |  | Theoretical results |  |
| --- | --- | --- | --- | --- |
| Z0(ohms) | α(neper/km) | β(radians/km) | α(neper/km) | β(radians/km) |
|  | 0.33 | 2.99 | 0.2378 | 4.421 |

THEORETICAL RESULTS:

SIMULATION RESULTS:

TEST CASE 2:

Examine the impact of resistance and inductance on the significance of Characteristic Impedance (Zo) and Propagation Constant (γ) of a Transmission Line at a frequency f=2.5 Khz. Vary the resistance value from 10 Ω to 50 Ω at the interval of 10 Ω with L,G and Cvalues constant and calculate the Characteristic Impedance and Propagation Constant .Vary the inductance values from 3.5 mH to 11.5 mH at the interval of 2.5 mH with R,G and C values constant.

PROGRAM:

clear; clc;

R=10;L=0.003;f=2*10^3;G=4*(10^-6);C=16*(10^-6);

w=2*%pi*f;

Z=R+(%i*w*L);

Y=G+(%i*w*C);

Zo=sqrt(Z/Y);

C=round(real(Zo));

D=round(imag(Zo));

printf('-Zo = %f + j(%f)ohms\n',C,D);

P=sqrt(Z*Y);

a=real(P);

a1=round(a*10000)/10000;

printf('-Attenuation constant a = %f neper/km\n',a1);

b=imag(P);

b1=round(b*10000)/10000;

OUTPUT:

Variable R

Constants L=3mH; f=2000; G=.4*(10^-6); C=8*(10^-6).

| S.No | (R) (Ω) | Theoretical (Z_0) (Ω) | Theoretical α | Theoretical β | Simulation α | Simulation β |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 10 | 19.53 − j2.546 | 0.2560 | 1.9635 | 0.2560 | 1.9635 |
| 2 | 20 | 19.994 − j4.975 | 0.5002 | 2.0100 | 0.5002 | 2.0100 |
| 3 | 30 | 20.667 − j7.220 | 0.7258 | 2.0777 | 0.7258 | 2.0777 |
| 4 | 40 | 21.468 − j9.267 | 0.9316 | 2.1582 | 0.9316 | 2.1582 |
| 5 | 50 | 22.337 − j11.133 | 1.1192 | 2.2456 | 1.1192 | 2.2456 |

Variable L

Constants R=10Ω; f=2000; G=.4*(10^-6); C=8*(10^-6).

| S.No | (L) (mH) | Theoretical (Z_0) (Ω) | Theoretical α | Theoretical β | Simulation α | Simulation β |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 3.5 | 21.050 − j2.363 | 0.2375 | 2.1161 | 0.2375 | 2.1161 |
| 2 | 8 | 31.662 − j1.571 | 0.1579 | 3.1830 | 0.1579 | 3.1830 |
| 3 | 6 | 27.446 − j1.812 | 0.1822 | 2.7592 | 0.1822 | 2.7592 |
| 4 | 11 | 37.105 − j1.340 | 0.1348 | 3.7302 | 0.1348 | 3.7302 |
| 5 | 11.5 | 37.937 − j1.311 | 0.1318 | 3.8138 | 0.1318 | 3.8138 |

SIMULATION RESULT:

THEORETICAL RESULT:

Test Case 3:

Explore the impact of frequency and capacitance on the significance of Characteristic

Impedance (Zo) and Propagation Constant (γ) of a Transmission Line. Vary the frequency

from 2KHz to 10 KHz at the interval of 2 KHz with L,R ,G and C values constant and

calculate the Characteristic Impedance and Propagation Constant . Vary the capacitance

values from 8 μF to 16 μF at the interval of 2 μF with R,G and L values constant.

OUTPUT:

Variable f

Constants L=3mH; f=2000; G=.4*(10^-6); C=8*(10^-6).

| S.No | Input (f) (KHz) | Theoretical (Z_0) (Ω) | Theoretical α (neper/km) | Theoretical β (rad/km) | Simulation α (neper/km) | Simulation β (rad/km) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 19.532 − j2.546 | 0.2560 | 1.9635 | 0.2560 | 1.9635 |
| 2 | 4 | 19.407 − j1.281 | 0.2576 | 3.9021 | 0.2576 | 3.9021 |
| 3 | 6 | 19.384 − j0.855 | 0.2580 | 5.8460 | 0.2580 | 5.8460 |
| 4 | 8 | 19.376 − j0.642 | 0.2581 | 7.7914 | 0.2581 | 7.7914 |
| 5 | 10 | 19.372 − j0.513 | 0.2581 | 9.7373 | 0.2581 | 9.7373 |

Variable c

Constants R=10Ω; f=2000; G=.4*(10^-6); C=8*(10^-6).

| S.NO | INPUT (C) (μF) | Theoretical (Z_0) (Ω) | Theoretical α | Theoretical β | Simulation α | Simulation β |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 8 | 19.532 − j2.546 | 0.2560 | 1.9635 | 0.2560 | 1.9635 |
| 2 | 10 | 17.470 − j2.278 | 0.2862 | 2.1953 | 0.2862 | 2.1953 |
| 3 | 12 | 15.948 − j2.079 | 0.3135 | 2.4048 | 0.3135 | 2.4048 |
| 4 | 14 | 14.765 − j1.925 | 0.3387 | 2.5975 | 0.3387 | 2.5975 |
| 5 | 16 | 13.811 − j1.801 | 0.3620 | 2.7769 | 0.3620 | 2.7769 |

SIMULATION RESULT: