EXNO: 3

Designing a Quarter wave transformer by varying following

parameters

a) frequency

b) input impedance

c) characteristic impedance

Test Case 1:

Design a quarter-wave transformer for an input impedance (Zin) of 50 ohms and a

load impedance (ZL) of 100 ohms. Analyze the transformer's performance as the

operating frequency is varied across {100 MHz, 200 MHz, 300 MHz, 500 MHz}.

OUTPUT:

ZR=100 ohms Zin =50 ohms

| ZS ohms | ZR ohms | Ro ohms |
| --- | --- | --- |
| 100 | 50 | 70.71 |

ZR=100 ohms Zin =50 ohms

| Frequency (MHz) | Length of λ/4 transformer (meters) |
| --- | --- |
| 100 | 0.750 |
| 200 | 0.375 |
| 300 | 0.250 |
| 500 | 0.150 |

Simulation Results:

Theoretical Results

Test Case 2:

For a fixed operating frequency of 2 GHz and a load impedance (ZL) of 100 ohms,

design a quarter-wave transformer and evaluate its performance as the input

impedance (Zin) varies through {25 ohms, 50 ohms, 75 ohms, 100 ohms}. Validate

that the transformer's characteristic impedance (Ztransformer= Ro= sqrt(Zin⋅ZL)

changes with Zin.

OUTPUT:

ZR=100 ohms Variable: Zin

| Zin | Ro |
| --- | --- |
| 25 | 50 |
| 50 | 70.71 |
| 75 | 86.60 |
| 100 | 100.00 |

Simulation Results: